RDP1000 “Remote Data Platform” User Manual

Version 2.01

October, 2014

Preliminary

Manufacturer: Advanced Telemetry Systems, Inc.

470 First Avenue North

Isanti, MN 55040

763-444-9267

www.atstrack.com

1.0 Functionality

The RDP1000 is designed to record hourly data dumps from the RS232 data port of an ATS R4500 Series Receiver/Datalogger and send that data via the Iridium satellite network’s RUDICS data service. This data will be received by a server that decompresses the data and sends it to you via a previously assigned email address (optional) and Dropbox folder. This system has the advantage of working in remote locations.

To use the RDP1000 you first need to have a Dropbox account and set up an airtime account with ATS. Also have ATS activate your SIM card and let them know your email address and if you want to receive a daily email of the transmitted data.

The RDP1000 uses 12 Volt dc power and makes use of a junction box which is shown connected to the major system components in the following photo:

The RDP1000 should be installed so that the black patch antenna is orientated at the top of the box where it will be exposed to a clear view of the sky and the bottom should be placed so that the status light indicators can be clearly seen.

The “Light Indicators” are

- Red – The RDP1000 is powered.

- Yellow – The RDP1000 data buffer is receiving data from the R4500.

- Green – The RDP1000 has the Iridium satellite modem powered up for either transmitting data or checking on satellite signal reception quality.

The PC can be temporarily attached during a site visit to check on the RDP1000 functionality via a USB cable and interface software provided by ATS. If the USB connection should hang up, unplug and re-plug the connection to reestablish communication.

2.0 Transmit/Listening Schedule

The RDP1000 receives data from R4500 that is dumped at the top of each hour for the previous hour’s data. At some time later the RDP1000 will transmit this data up to the Iridium’s RUDICS network. If the data is unable to be picked up the RDP1000 will continue to make attempts until either it is successful or it needs to become ready for the next hour’s data from the R4500. A copy of this data will be maintained in the R4500 and the RDP1000 until the memory of the devices rolls over and starts to be overwritten.

In order to save on data costs the RDP1000 has the ability to shut down certain hours and/or days that it receives/transmits data from the R4500. This is programmed by using the RDP1000’s interface software.

3.0 Interface Software

The ATS RDP1000 interface software can be downloaded from the atstrack.com web site or if requested it can be installed from a compact disk sent along with the RDP1000.  The software is compatible with Windows XP, Vista, Windows 7 and Windows 8 operating systems.  Load the CD, click on the setup executable file  and follow the instructions.

USB Driver Installation: The following dialog box will appear when running the software the first time after it has been installed:

Selecting the correct environment followed by clicking OK will install the driver that will enable USB communication with the RDP1000.

3.1 Select Device:

The first or second screen that appears when the software is run is shown below:

Select from the dropdown list the serial number of the RDP1000 and click OK.

3.2 Main Command Window:

The following main screen will appear displaying a command box:

3.3 View/Edit Configuration:

This function accessed by the USB connection allows access to the RDP1000’s clock, scheduling and version information. Upon entering this screen, the RDP1000 will also enter a special timekeeping mode so that it can continuously update the time portion of the display in realtime.

3.3.1 Transmit/Listening Schedule

To determine the schedule the RDP1000 will take data in from the R4500 or send data up to the Iridium’s RUDICS network, click one either the “Input Device Listening Schedule” or the “Data Output Transmit Schedule” buttons. Once in that screen clicking on the buttons next to the hour or day will activate/deactivate that specific hour or day. Clicking on the headers will activate/deactivate the whole group of hours or days. In the screen pictured below the RDP1000 will not record data from the R4500 on Oct 19-21 and on the rest of the days it will not record data from the R4500 on just the hours 9-12. To see the rest of the schedule for the year click on the Red or Green arrows to advance through the months.

After exiting this screen the schedule is still not sent over to the RDP1000. Clicking on the Send to RDP button will send over the schedule information along with the PC’s time and date over to the RDP1000, synchronizing the two clocks.

3.4 Test Signal Strength:

This function accessed by the USB connection allows testing the Iridium satellite signal level. This signal level can change from moment to moment. What the RDP1000 and its interface software can do is display snapshots of it. The display will show 5 possible signal strength rectangles. The rectagles will fade out as the signal weakens. A timestamp at the bottom of the display will show the time of the last signal strength value reported. While in this mode the RDP1000 is constantly testing for a signal level but each measurement takes time to access and report. The RDP1000’s green status indicator light will be turned on. While in this mode the RDP1000 is performing no other activity and will need to exit this mode by cycling the RDP1000’s power.

3.5 Offload Data:

The RDP1000 makes a backup of the data it has been given to send that can be offloaded later by acessing this function and selecting the green button. The data gets saved to a file with the following name format “c:\Advanced Telemetry Systems, Inc\ATS RDP1000\Data\RDP_21826PM_7282014.bin”. This file is in the untranslated format.

3.5.1 Convert External Data Output File:

To translate the offloaded file: select “Convert External Data Output File” from the Main Screen’s file dropdown.

Which provides the following dialog box:

The converted file will be created after clicking on the “Translate Data” and providing a file name to convert.

3.6 Data Format

The data sent by the R4500 to the RDP is compressed in order to minimize the satellite data charges. The data is decompressed before sending to your email and/or Dropbox account. If information is lost in transmission that might also impact the ability to decompress some of the successfully sent data. The data below are sample sets of data that could be sent to your account. The two different samples are representative of the R4500C and the R4500S using the Standard Pulse Rate option.

3.6.1 Transmission Header Format

(e.g. “13010 137/00:00 12.6 RUDICS 150c”)

| Contents | Description |
| --- | --- |
| Receiver Serial Number | A five character serial number that designates the year of receiver production and three characters that designate sequential production number (e.g. “13010”) |
| Day | A number from 1-366 representing the Julian day (1-366 for a leap year, otherwise 1-365). This indicates the day the data was processed for transmission (e.g. “137”) |
| Hour | A number from 0-23 representing the hour. This indicates the hour the data was sent by the R4500 over to the RDP1000 for transmission. (e.g. “00:00”) |
| Volts | Voltage to the tenths of volts representing the system voltage at the time the data was sent over to the RDP1000 (e.g. “12.6”) |
| Compression | Compression format information that the translation software used to decompress the data to an understandable format. (e.g.”RUDICS 150c” or “RUDICS 148s Fixed:2-15) |

3.6.2 Data File Format

| Column Name | Description |
| --- | --- |
| Day: | A number from 1-366 representing the Julian day (1-366 for a leap year, otherwise 1-365). This appears in the transmission header above and it indicates the day the data was processed for transmission. |
| Hr: | A number from 0-23 representing the hour. This appears in the transmission header above and it indicates the hour the data was collected. The data is sent from the R4500 at the top of the hour over to the RDP1000. If the transmissions are keeping up with the data collection this will be one hour before the time it is transmitted. |
| Volts: | Voltage to the tenths of volts representing the system voltage at the time the data was sent over to the RDP1000. This appears in the transmission header above. |
| Log Type: | (Used for the R4500S only) Indicates what type of logging is being done. Fixed–NN: The pulse rate is fixed and the noise multiplier is NN. The noise multiplier is applied to the noise indicator to determine the range of noise pulses being received. The noise multiplier used is determined by the maximum number of noise pulses allowed for a data point to be transmitted to the RDP1000. Noise Multiplier Range of Maximum Noise Pulses 35 211-255 30 176-210 25 141-175 20 140-71 10 36-70 5 15-35 2 8-14 The default noise multiplier is 35. The example data set is using 15. The values shown below are for the noise multiplier 35. Noise Indicator Noise Pulses - 0 - 1-35 - 36-70 - 71-105 - 106-140 - 141-175 - 176-210 - 211-255 |
| Freq: | A number representing the transmitter’s frequency. It will be a number from 0-9999 representing the last four digits of the frequency. |
| Code: | (Used for the R4500C only) The number representing the code of the detected transmitter. |
| Pulse Rate: | (Used for the R4500S only) The pulse rate of the incoming signal. |
| Ant#: | A number from 0-8 representing the antenna being used. A ‘0’ indicates all antennas were turned on. |
| Signal: | A number from 40-154 representing the signal strength. |
| Hr: Min: | A number representing the hour and minute this data point was saved to memory. |
| #Matches: | A number from 1-255 representing the number of codes detected during the scan. |
| Mort: | (Used for the R4500C only) One of three possibilities: “yes”, “no” or “?”. To indicate the presence of a mortality pulse. A “?” indicates the mortality pulse is not being picked up consistently so it could be noise instead. |
| Noise Ind: | (Used for the R4500S only) Used on fixed pulse rate logging. It indicates level of noise. See Log Type for further explanation. |
| END: | The END appears at the end of the data sent to the RDP. If it does not appear you know the data transmission was incomplete and some data was lost. (Used for the R4500S only) The ### following the END indicates the number of data blocks skipped over for this transmission. This number can be nonzero only if you choose in your R4500S configuration not to send data that had noise pulses exceeding a preset limit. |