SR3017 Trident Acoustic Receiver, Shore-based

User Manual

Version 3.0

Advanced Telemetry Systems, Inc.

470 First Avenue North

Isanti, MN 55040

763-444-9267

www.atstrack.com

TABLE OF CONTENTS

1.0 Functionality	4

2.0 Start-up and LED Sequences	6

3.0 Data File Format	8

3.1 Header Format	9

3.2 Data Format	10

4.0 Trident Receiver USB Interface and Filter Software	11

4.1 Select Sonic Receiver (Change Receiver)	11

4.2 Main Command Window	12

4.3 Edit Configuration	13

4.4 View Real Time Logging	14

4.5 GPS Data Tagging for Mobile Tracking	17

4.6 Filter Data	19

4.6.1 Standard JSAT’s Coded Tags	19

4.6.2 Temperature and Depth Tags	20

4.6.2.1 Barometric Pressure	20

4.6.2.2 Depth Temperature Tag Code List	21

5.0 Filter Data File Format	21

5.1 Filter File Output Type 1	21

5.2 Filter File Output Type 2	22

5.3 Filter File Output Type 3	25

5.4 Filter File Output Type 4	26

5.5 Filter File Output Type 5	26

5.6 Additional Output for Temperature and Depth Tags	28

5.6.1 Data Appended to Filter File Output Type 2	28

5.6.2 Data Appended to Filter File Output Type 4	30

5.6.3 Data Appended to Filter File Output Type 5	31

5.6.4 Additional Filter File Output Type 6	32

6.0 Triton’s Remote Communicator - Remote Offload Tool	35

6.1 Main Command Window	35

6.2 Remote Connect via TCP/IP	36

6.2.1 Sierra Wireless RV50 Modem Configuration Settings	36

6.2.2 Telnet Screen	37

6.2.3 Get Directory	37

6.2.4 Offload file data and save to PC	38

6.2.5 Start Unattended Logging	38

6.3 Direct Connect via Serial Port	39

6.3.1 Get Directory	40

6.3.2 Offload file(s)	40

6.3.3 Unattended Logging	41

7.0 Addendum: Remote Control via Cellular Modem	42

# 1.0 Functionality

The autonomous node receiver is designed to be a self-sufficient, data-logging unit that sits above the water line with a hydrophone and temperature cable anchored to the bottom of marine and freshwater environments.  The major components of the receiver system are shown in Figure 1-1.

Figure 1-1

The hydrophone receives the high frequency mechanical vibrations sent through the water by the JSATS transmitter (in fish) and converts them to weak electrical voltages.  These weak voltages are amplified and filtered by the preamplifier of the Control/Power board (to reduce noise) and then sent to the Digital Signal Processor (DSP) for processing.

The DSP converts the incoming filtered signals to digital numbers for use by the DSP in its detection and decoding algorithm.  The detection algorithm looks for the existence of a tag and the decoding algorithm determines what specific tag code is present.

When a valid code is verified by the DSP it passes the detection information on for storage to the SDHC (High capacity SD flash memory) card.  The supervisory processor on the board manages the clocks, sensor measurements, data storage and the communication with the external USB and RS232 connections.

The receiver is equipped with sensors for temperature, tilt (optional), pressure (optional) and GPS. The GPS is used to obtain accurate timing information for setting and syncing the multiple clocks. This is critical in providing accuracy for timestamping the signal arrival times.  If the temperature, tilt and pressure sensors are not enabled the data recorded will be displayed as “N/A”.  The receiver queries the sensors and voltage approximately every 15 seconds. If no tags are present this data will be saved and written to the flash card as a dummy tag data approximately every 10 minutes.

The receiver is equipped with an USB port that can be used to see real-time data.  This port can be accessed from the top edge of the housing by unscrewing the cover on the connector with the USB label. It uses a standard USB cable. If the USB connection should hang up, unplug and re-plug the connection to reestablish communication.

The receiver is equipped with a RS232 serial port that can be used to pull or push data stored on the SDHC memory card. This is handy for connecting to a cell modem or something similar for use in getting your data remotely. This can be done with or without additional software provided by ATS. The com port settings are 115200, N, 8, 1 with no hardware handshaking. To see the available commands send a <CR> to trigger a prompt then enter a <?>. This port can be accessed from the top edge of the housing by unscrewing the cover on the connector with the RS232 label. It uses an optional serial cable for use with a cell modem or a serial cable for use with a PC provided on request. Make sure to specify the intended use.

The receiver is powered using a 12V power supply such as a deep cycle marine battery or solar panel.

Notes:

- The power consumption of the receiver is ~40 milliamps during normal operation.

- The recommended SDHC flash card is the SanDisk with a capacity of 32GB or smaller.

Important Note: Make sure the flash card has been formatted using the Default format options. The file system usually will be FAT32. DO NOT format using the quick format option.

3.   A card reader (not supplied) is required for the SDHC card.

# 2.0 Start-up and LED Sequences

With the housing open, place a SDHC flash card in the slot.  The rest of the connections are attached to the outside of the box without needing to open the housing. Connect the power cable into the port connector located on the left bottom of the housing. The middle connector is for the temperature sensor and the connector on the right is for the hydrophone. See Figure 1-1.  Observe the different status LEDs to understand what operation is taking place.  The yellow GPS status LED flashes when the GPS board is powered and no lock on a fix has been obtained. The blue SDHC LED will turn on whenever the flash card is being read from or written to.  The main unit status LEDs are described in Table 2-1 below.

| Sequence | Yellow LED | Green LED | Red LED | Event | Description |
| --- | --- | --- | --- | --- | --- |
| Initialization Sequence |  |  |  |  |  |
| 1 | On | On | On | Power Up |  |
| 2 | Off | On | On/Off | Reading Sensors, checking voltage … | Flashing Red. |
| 3 | On or On/Off | Off | On or On/Off | Clock calibration and time sync | If a GPS signal is present it will sync every 5 minutes. |
| Misc. |  |  |  |  |  |
| x | Off | On | Off | In the time keeping routine will not store tag detections. Do a power cycle to cancel. | Continuous solid green |
| Main Routine |  |  |  |  |  |
| x | Off | On | On | DSP Reset Scheduled | Red and Green on together |
| x | On/Off | Off | Off | GPS timing pulse is present | Flashing Yellow |
| x | On/Off | On/Off | On/Off | SDHC flash card not inserted in slot | All three LEDs flash together |
| x | Off | On | On/Off | Reading sensors and voltage values | This happens every fifteen seconds. Will flash Red while Green is on if a sensor is disconnected or malfunctioning |
| x | Off | Off | On | Tag data decoded | Flashes for the first 2400 detections then quits |

Table 2-1

# 3.0 Data File Format

All tag detections are stored in “.csv” files that can be read directly by most text editors such as Microsoft’s “Excel” and “Notepad”.  Files can be created hourly, daily or the default as one solitary file. On power up the receiver is set up to use only one file. It can be changed by receiving commands through its RS232 serial port. The commands “HON” and “DON” will trigger hourly file creation. The command “DAYON” will trigger daily file creation. The hourly and daily files are useful in remote file dumping since the files being dumped are smaller sizes. The filenames consist of the serial number and creation timestamps. The naming conventions are listed below:

- SR17034_yymmdd_hhmmss.csv  - Single name file format

- SR17034Hyymmdd_hhmmss.csv - Hourly file name format

- SR17034Dyymmdd_hhmmss.csv – Daily file name format

A snippet of an example data file is shown in Figure 3-1.

Figure 3-1

## 3.1 Header Format

Table 3-1 gives a description of the information contained in lines 1-10 shown in Figure 3-1.

| Line Contents | Description |
| --- | --- |
| Site/System Name | Descriptive name defined by the user and separated by two commas (e.g. “ATS, NC, 02). |
| File Name | 8 character site name which consists of “SR” followed by the serial number then a “_”, ”H”, or “D” depending on whether it is a single, hourly or daily type file. This is followed by date and time of file creation (e.g. “SRser##_yymmdd_hhmmss.csv”) |
| Receiver Serial Number | A five character serial number that designates the year of receiver production and three characters that designate sequential production number (e.g. “17035”) |
| Receiver Firmware Version | The name and version of the receiver supervisory firmware and the name. |
| DSP Firmware Version | The name and version of the DSP firmware. |
| File Format Version | Version number of the file format |
| File Start Date | Date and time signal acquisition began (mm/dd/yyyy hh:mm:ss hhz) The 2 digit number proceeding the letter ‘z’ is the offset from Coordinated Universal Time (UTC). For example Figure 3-1 has ‘-05z’ which is UTC-5 hours or Central Daylight Savings Time. |
| File End Date | Date and time signal acquisition ended (mm/dd/yyyy hh:mm:ss) Appears at the end of the data set. |

Table 3-1

## 3.2 Data Format

Table 3-2 gives a description of the columns listed in line 11 shown in Figure 3-1.

| Column Name | Description |
| --- | --- |
| Internal | Diagnostic and timing information. Data here will vary depending on the version. |
| SiteName | Descriptive name defined by the user and separated by two commas (e.g. “ATS , NC, 02”). |
| DateTime | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss) |
| TagCode | 9 digit tag code as decoded by receiver (e.g. “G720837eb”) G72ffffff is used as a dummy tag for data recorded when no tag is present. Also one line of text:“Old Clock” followed by a line of text: “New Clock” will appear in this field when the configuration window sends over a new time. |
| Tilt | Tilt of the receiver (degrees). This typically will appear as “N/A” since this sensor is normally not included. |
| VBatt | Voltage of the receiver batteries (V.VV). |
| Temp | Temperature (C.CCº). |
| Pressure | Pressure outside of receiver (absolute PSI). This typically will appear as “N/A” since this sensor is normally not included. |
| SigStr | The logarithmic value for signal strength (in DB) “-99” signifies a signal strength value for an absent tag |
| BitPeriod | Optimal sample rate at 10 M samples per sec. To convert to frequency in kHz divide into 100,000. |
| Threshold | The logarithmic measurement of background noise used for tag detection threshold. |

Table 3-2

# 4.0 Trident Receiver USB Interface and Filter Software

The ATS Trident Receiver USB interface and filter software can be downloaded from our website. The software is compatible with Windows 7 and Windows 10 operating systems.  After downloading the software click on the setup executable and follow the instructions.

USB Driver Installation:  The Trident software will walk you through installing the USB driver on its first boot up. If it is not done here the USB driver will need to be installed as a separate step. The driver installation can be initiated by going into the Settings menu of the main command window and selecting Install Driver.

## 4.1 Select Sonic Receiver (Change Receiver)

The first screen that appears when the software is run is shown Figure 4-1.

Figure 4-1

The USB Communication mode allows for real-time data viewing while a computer is attached to the USB port.  Enter the serial number of the receiver. This can be found on the placard affixed to copper shielding easily viewed through the clear window of the receiver’s housing and click OK.

## 4.2 Main Command Window

Next, the Main Command window appears as shown in Figure 4-2.

Figure 4-2

The USB connection allows you to update the receiver’s configuration - Edit Configuration and view the tags as they are being decoded - View Realtime Logging.

## 4.3 Edit Configuration

Figure 4-3

This function accessed by the USB connection allows access to the Trident receiver’s configuration. Upon entering this screen, the receiver will also enter a special timekeeping mode so that it can continuously update the time portion of the display in realtime. While in this mode  the green status LED will be lit continously.

To update the time and date on the receiver so it matches the PC’s, click on the blue button Set Receiver Clock to PC Clock, and the PC time and date will be sent to the Trident receiver, synchronizing the two clocks.

The offset from Coordinated Universal Time (UTC) will also be sent and stored on the receiver but to set that click on the green button Send to Receiver. If the Local Time check box is not clicked UTC time will be used.

When the Trident receiver updates its clock it will send to the SDHC card two lines of data. The first represents the time of the update using the old time, and the second the time of the update using the newly corrected time. The Site Name for the SR3017 is fixed. It will be “SR” followed by the receiver serial number. The Site/System Name is customizable and will be sent over as it appears on the screen but is done as a separate step by clicking on the green button Send to Receiver located at the bottom of the screen. When finished, make sure to click on the red Close button so the receiver will get the command to exit the timekeeping mode. Otherwise, cycling the  power on the receiver will accomplish the same thing.

The time setting here will be overwritten by GPS time on boot up or when the hour rolls over if it is able to get the information from the satellite constellation. If you will be running these units where you will have access to GPS then you only need to do this configuration step once. This step will save the time zone stored on your PC which will allow your GPS sync’d timestamps to appear as local time. The GPS sync’d time will never be in daylight savings time. Using the GPS to set the clock provides improved accurate time syncing across different SR3017 units.

## 4.4 View Real Time Logging

Figure 4-4

You may view real time datalogging of tag data using the USB connection by selecting the View Realtime Logging button, and then selecting the green Start button at the bottom of the screen. This displays the data as it is being captured by the Trident Receiver.

If the SDHC card is present in the SD card slot of the receiver, data will appear in blocks of fifteen seconds of accumulated data, with data appearing every 15 seconds on the screen.

If the SD card slot is empty, the data will be displayed immediately as it is detected.

Over time this data will develop a time lag depending on the amount of data being printed to the screen and the speed of the PC.

Figure 4-5

The View Realtime Logging function has a number of display options to facilitate viewing the incoming data. These options can be selected from the Settings drop-down menu at the top of the screen, as shown n Figure 4-5. For example, detections can be shown as separate lines of data, as shown in Figure 4-4, or by using the Summarize Data option.

The Summarize Data option will display one data line per tag. The screen is refreshed for each new data point. It can be selected to filter detections having periods too large or too small to be valid. This option is shown below in Figure 4-6 and Figure 4-7.

Figure 4-6

Figure 4-7

This screen also shows in the farthest left column a speaker icon followed by a column of check boxes. If a tag code is checked it will play a tone that will be tied to it’s last signal strength value. It will change the tone’s pitch and duration accordingly. Since playing the tone pauses the operation momentarily it will slow screen updates down a bit. Ideally keep the number of boxes checked to a small number.

## 4.5 GPS Data Tagging for Mobile Tracking

When mobile tracking and logging data to the PC there is an option to read in serial data from a GPS unit or a special SR3017 “Mobile” version receiver. The RS232 serial data out must be streaming the NMEA $GPRMC sentences at a baudrates of 4800, 9600, or 115200. The special SR3017 “Mobile” version receiver uses 115200 and streams the GPS data out its port with the RS232 label. On request an optional PC serial cable is provided for the receiver. Make sure to enter the baudrate and com port used under the “Settings” Tool bar option as shown in Figure 4-8. If you are using a GPS receiver instead of the special SR3017 “Mobile” version receiver make sure it is using the following settings: Data – 8 bit, Parity – none, Stop bits – 1 and no flow control.

Figure 4-8

When the logging with GPS option is selected the logging screen will look something like Figure 4-9.

Figure 4-9

At the conclusion of the logging session the data will be found on the PC in the directory: “> Local Disk (C:) >Advanced Telemetry Systems, Inc > ATS Trident Receiver > Log”.

A snippet of an example Log file containing GPS data is shown in Figure 4-10.

Figure 4-10

## 4.6 Filter Data

Figure 4-11

This option does not make use of an active USB connection. It takes as input one or more of the Trident Receiver files residing on your computer that have been copied over from the SDHC card(s). It post processes the data by filtering out invalid data, splitting the files into smaller chunks and summarizing run data.

### 4.6.1 Standard JSAT’s Coded Tags

For standard JSAT’s coded tags, there are two filtering methods to choose from. They give slightly different results. Method “A-Default” and method “B-Minimum Mode”.

Method “A” (Default - SVP) looks for tags with consecutive repeating periods that are within a certain range of the selected nominal period(s). These periods need to stay within a narrow range of each other.

Method B developed by Pacific Northwest National Laboratory (PNNL) uses a moving window. The window size is about 12 times the estimated pulse rate interval. In this window the tag period used is the minimum mode value close to the nominal.  Both of these routines can take a while to process all the data. It does allow a number of files to be processed at a time. As it processes, the data summary information will be displayed.

Before starting the routine, make sure to check the boxes next to the periods of the sonic transmitters you used. Figure 4-11 shows a period of 3 seconds checked.

### 4.6.2 Temperature and Depth Tags

ATS manufactures in addition to standard JSAT’s coded tags, tags that transmit the JSATs code along with the tag’s current temperature and/or depth. This data can be retrieved and deciphered by clicking on the check box located at the bottom of the screen shown in Figure 4-11. This option is only available using Filter Method “A-Default”.

Processing the temperature and depth tag data will require additional input into the filter program.

#### 4.6.2.1 Barometric Pressure

Depth measurement is really a measurement of pressure. To calculate depth the local barometric pressure needs to be taken into account. This pressure frequently changes, but the filter can only use one value for its depth calculation. Pick a midrange value that is fairly representative of the site’s average barometric pressure during the time the data was collected.

The value entered can be designated in units of atmospheres (atm), mercurial inches (inHg), kilopascals (kPa), millibars (mBar), mercurial millimeters (mmHg), or pounds per square inch (psi). Ensure that the correct type of units are selected or else incorrect results will be calculated.

Figure 4-12

#### 4.6.2.2 Depth Temperature Tag Code List

A simple “.csv” file is needed for input containing a list of the temperature and depth tag codes that were deployed. Below is what the contents of a possible file would look like…

G724995A7

G724D5B49

G72453398

G72452BC7

G724A9193

G722A9375

G724BA92B

G724A2D02

# 5.0 Filter Data File Format

When the filter option from the File Data dialog is finished running there will be a number of new files created. They will consist of 5 different types.

Example input file name:

SR17102_171027_110750.csv

One example each of the 5 types of output files:

Type 1) SR17102_171027_110750_Log1_1027_1107_2.csv

Type 2) SR17102_171027_110750_DData_Log1_1027_1107_2.csv

Type 3) SR17102_171027_110750_RejectedTags_Log1_1027_1107_2.csv

Type 4) SR17102_171027_110750_Cleaned_Log1_1027_1107_2.csv

Type 5) SR17102_171027_110750_summary_Log1_1027_1107_2.csv

## 5.1 Filter File Output Type 1

Example Type 1 output file names:

SR17102_171027_110750_Log1_1.csv

SR17102_171027_110750_Log1_1027_1107_2.csv

SR17102_171027_110750_Log2_1027_1110_1.csv

SR17102_171027_110750_Log2_1027_1110_2.csv

.

.

.

The input file can contain multiple logging sessions which are defined to be a power on off or the insert and removal of a SDHC card. The input file can be larger than some programs like Excel can handle. Type 1 files are partitioned copies of the input file. These partitions isolate data into files according to the log session and they keep the files smaller than 50,000 lines of data.

## 5.2 Filter File Output Type 2

Example Type 2 output file names when the “A – Default” selection in the File Data dialog was selected:

SR17102_171027_110750_DData_Log1_1027_1107_1.csv

SR17102_171027_110750_DData_Log1_1027_1107_2.csv

SR17102_171027_110750_DData_Log2_1027_1110_1.csv

SR17102_171027_110750_DData_Log2_1027_1110_2.csv

.

.

Example Type 2 output file names when the “B – Minimum Mode” selection in the File Data dialog was selected:

SR17102_171027_110750_MData_Log1_1027_1107_1.csv

SR17102_171027_110750_MData_Log1_1027_1107_2.csv

SR17102_171027_110750_MData_Log2_1027_1110_1.csv

SR17102_171027_110750_MData_Log2_1027_1110_2.csv

.

.

.

Type 2 files have all the information of the Type 1 files with additional information added on. This file will not include rejected data if the filter was run with the

Remove Filtered Hits from Final Data checkbox checked from the File Data dialog.

Figure 5-1

| Column Name | Description |
| --- | --- |
| Detection Date/Time | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss) |
| TagCode | 9 digit tag code as decoded by receiver (e.g. “G7280070C”) G72ffffff is used as a dummy tag for data recorded when no tag is present. |
| RecSerialNum | A five character serial number that designates the year of receiver production and three characters that designate sequential production number (e.g. “18035”) |
| FirmwareVer | The version of the receiver supervisory firmware. |
| DspVer | The version of the DSP firmware. |
| FileFormatVer | Version number of the file format. |
| LogStartDate | Date and time signal acquisition began for this logging session (mm/dd/yyyy hh:mm:ss) |
| LogEndDate | Date and time signal acquisition finished for this logging session (mm/dd/yyyy hh:mm:ss *####+mmddhhmmss) |
| FileName | Diagnostic and timing information. Data here will vary depending on the version. |

Table 5-1

Figure 5-2

| SitePt1 | Site name part 1. Descriptive name defined by the user. |
| --- | --- |
| SitePt2 | Site name part 2. Descriptive name defined by the user. |
| SitePt3 | Site name part 3. Descriptive name defined by the user. |
| Tilt | Tilt of the receiver (degrees). This typically will appear as “N/A” since this sensor is normally not included. |
| VBatt | Voltage of the receiver batteries (V.VV). |
| Temp | Temperature (C.CCº). |
| Pressure | Pressure outside of receiver (absolute PSI). This typically will appear as “N/A” since this sensor is normally not included. |
| SigStr | The logarithmic value for signal strength (in DB) “-99” signifies a signal strength value for an absent tag |
| BitPrd | Optimal sample rate at 10 M samples per sec (related to tag frequency) |
| Threshold | The logarithmic measurement of background noise used for tag detection threshold. |
| ImportTime | Date and time this file was created (mm/dd/yyyy hh:mm:ss) |
| TimeSince LastDet | Elapsed time in seconds since the last detection of this code. |
| Multipath | Yes/No value indicating if the detection was from a reflected signal. |
| FilterType | SVP (Default)/ MinMode value indicating the choice of filtering algorithm used on this data. |
| Filtered | Yes/No value indicating if this data has been rejected. |
| NominalPRI | The assumed programmed value for the tag’s pulse rate interval. |

Table 5-2

Figure 5-3

| DetNum | The current detection number for this accepted code, or if followed by an asterisk, the count of previously rejected hits for this code. |
| --- | --- |
| EventNum | This count increases if there is a reacquisition of this code after an acquisition loss. For the SVP method this loss needs to be >= 30 minutes. For MinMode an acquisition loss happens if there are less than 4 hits contained within an acceptance window of 12 nominal PRIs. |
| EstPRI | The estimated PRI value. |
| AvePRI | The average PRI value. |
| ReleasedDate |  |
| Notes |  |

Table 5-3

## 5.3 Filter File Output Type 3

Type 3 files have the detection data for rejected codes.

Example Type 3 for the Default SVP filter output file names:

SR17102_171027_110750_RejectedTags_Log1_1027_1107_1.csv

SR17102_171027_110750_RejectedTags_Log1_1027_1107_2.csv

SR17102_171027_110750_RejectedTags_Log2_1027_1110_1.csv

SR17102_171027_110750_RejectedTags_Log2_1027_1110_2.csv

.

.

.

## 5.4 Filter File Output Type 4

Type 4 files are Type 1 files with the invalid tag detections removed.

Example Type 4 output file names:

SR17102_171027_110750_Cleaned_Log1_1027_1107_1.csv

SR17102_171027_110750_Cleaned_Log1_1027_1107_2.csv

SR17102_171027_110750_Cleaned_Log2_1027_1110_1.csv

SR17102_171027_110750_Cleaned_Log2_1027_1110_2.csv

.

.

.

## 5.5 Filter File Output Type 5

Example Type 5 output file names:

SR17102_171027_110750_summary_Log1_1027_1107_1.csv

SR17102_171027_110750_summary_Log1_1027_1107_2.csv

SR17102_171027_110750_summary_Log2_1027_1110_1.csv

SR17102_171027_110750_summary_Log2_1027_1110_2.csv

.

.

.

Type 5 files have the synopsis of data contained in the earlier files.

Figure 5-4

| Column Name | Description |
| --- | --- |
| First Date/Time | Date and Time of first acquisition of the listed Tag Code. Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss) |
| Last Date/Time | Date and Time of last acquisition of the listed Tag Code. Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss) |
| Elapsed | Time difference in seconds between the first two columns. |
| Tag Code | 9 digit tag code as decoded by receiver (e.g. “G7229A8BE”) |
| Det Num | The number of valid detections for the listed tag code. If an “*” is present the Tag Code was filtered out as a false positive. |
| Nominal | The assumed programmed value for the tag codes’ pulse rate interval. |
| Ave | The average PRI value. An adjacent “*” indicates it was > then 7 periods long. |
| Est | The estimated PRI value. |
| Smallest | The smallest PRI that was a valid value. The PRIs checked off in the File Data dialog are used to determine the set of acceptable PRIs. |
| Largest | The largest PRI that was a valid value. The PRIs checked off in the File Data dialog are used to determine the set of acceptable PRIs. |
| Sig Str Ave | The average signal strength of the valid data for the listed tag code. |
| Min Allowed | Lower Signal strength values are filtered out. |
| # Filtered | Number of acquisitions for the listed tag code that have been filtered out. |

Table 5-5

## 5.6 Additional Output for Temperature and Depth Tags

When the filter is done running there will be the same output as with running without the temperature depth tag option selected with a few additions.

One additional file type…

Type 6) SR17102_171027_110750_SensorTagData_Log1_1027_1107_2.csv

And additions to the following file types…

Type 2) SR17102_171027_110750_DData_Log1_1027_1107_2.csv

Type 4) SR17102_171027_110750_Cleaned_Log1_1027_1107_2.csv

Type 5) SR17102_171027_110750_summary_Log1_1027_1107_2.csv

### 5.6.1 Data Appended to Filter File Output Type 2

The following is an example of the data appearing as additional columns appended to the dataset after the column labeled “Notes”.

Figure 5-5

| Column Name | Description |
| --- | --- |
| SensorTag | Character denoting general sensor information as defined below… N - Detection information is for a non-sensor tag. Y - Detection information is for a sensor tag but no sensor data was paired with this detection. T – Detection information is for a sensor tag and is paired with temperature data only. D- Detection information is for a sensor tag and is paired with depth data and possibly temperature data. |
| TempDateTime | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss). This timestamp is for the received code imparting a tag’s temperature information. |
| TempSensorCode | 9 digit tag code as decoded by receiver (e.g. “G7207975C”) representing the temperature information. |
| TagTemp(C) | The temperature (C.CCº) measured by the sensor tag. |
| DepthDateTime | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss). This timestamp is for the received code imparting a tag’s depth information. |
| DepthSensorCode | 9 digit tag code as decoded by receiver (e.g. “G720B3B1D”) representing the depth information. |
| TagPress(mBar) | The pressure (PPPP.P) in mBar measured by the sensor tag. |
| TagDepth(m) | The converted depth position (DDD.DD) in meters measured by the sensor tag. |
| SensorPrd | The period of the sensor codes in seconds appearing after the primary code. |

Table 5-5

### 5.6.2 Data Appended to Filter File Output Type 4

The following is an example of the data appearing as additional columns appended to the data after the column labeled “Threshold”.

Figure 5-6

| Column Name | Description |
| --- | --- |
| Temperature Date/Time | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss). This timestamp is for the received code imparting a tag’s temperature information. |
| Temp SensorCode | 9 digit tag code as decoded by receiver (e.g. “G7207975C”) representing the temperature information. |
| Tag Temp(C) | The temperature (C.CCº) measured by the sensor tag. |
| Depth Date/Time | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss). This timestamp is for the received code imparting a tag’s depth information. |
| Depth SensorCode | 9 digit tag code as decoded by receiver (e.g. “G720B3B1D”) representing the depth information. |
| Tag Press(mBar) | The pressure (PPPP.P) in mBar measured by the sensor tag. |
| Tag Depth(m) | The converted depth position (DDD.DD) in meters measured by the sensor tag. |

Table 5-6

### 5.6.3 Data Appended to Filter File Output Type 5

This file has only one additional columns appended to it. It appears after the column labeled “# Filtered”. It is labeled “Sensor Tag” and just indicates whether the code listed belongs to a sensor tag with the indicator “Y” or “N”.

### 5.6.4 Additional Filter File Output Type 6

Example Type 6 output file names:

SR17102_171027_110750_ SensorTagData _Log1_1027_1107_1.csv

SR17102_171027_110750_ SensorTagData _Log1_1027_1107_2.csv

SR17102_171027_110750_ SensorTagData _Log2_1027_1110_1.csv

SR17102_171027_110750_ SensorTagData _Log2_1027_1110_2.csv

.

.

.

Type 6 files have the just the code, temperature and depth data broken down by the time the data was received.

Figure 5-7

| Column Name | Description |
| --- | --- |
| Tag Code Date/Time | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss) |
| TagCode | 9 digit tag code as decoded by receiver (e.g. “G7229A8BE”) |
| Secs | A decimal representation in seconds of the time the primary code was decoded. |
| Temperature Date/Time | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss) . This timestamp is for the received code imparting a tag’s temperature information. |
| TempCode | 9 digit tag code as decoded by receiver (e.g. “G7207975C”) representing the temperature information. |
| TempSecs | A decimal representation in seconds of the time the temperature code was decoded. |
| TempTimeSinceCode | The elapsed decimal time that has elapsed since the primary sensor tag’s code was detected. |
| Temp(C) | The temperature (C.CCº). measured by the sensor tag |

Table 5-7

Figure 5-8

| Column Name | Description |
| --- | --- |
| Depth Date/Time | Date recorded as mm/dd/yyyy. Time of detection, defined as the time the signal arrives at the hydrophone (TOA) and shall be recorded with microsecond precision (hh:mm:ss.ssssss) . This timestamp is for the received code imparting a tag’s depth information. |
| DepthCode | 9 digit tag code as decoded by receiver (e.g. “G720B3B1D”) representing the depth information. |
| DepthTimeSinceCode | The elapsed decimal time that has elapsed since the primary sensor tag’s code was detected. |
| DepthTimeSinceTemp | The elapsed decimal time that has elapsed since the temperature sensor tag’s code was detected |
| Press(mBar) | The pressure (PPPP.P) in mBar measured by the sensor tag. |
| Depth(m) | The converted depth position (DDD.DD) in meters measured by the sensor tag. |

Table 5-8

# 6.0 Triton’s Remote Communicator - Remote Offload Tool

Triton’s Remote Communicator facilitates remotely retrieving data from the SR3017 by simplifying the pushing and pulling of data from the stored data files. This is done via a PC’s serial port or a TCP/IP (internet) connection. This works through the RS232 serial port shown in Figure 1-1. The Triton’s Remote Communicator software is ideally set up to work with the Sierra Wireless Airlink RV50 cellular modem or directly connect to a PC’s serial com port. The software can be downloaded from our website and is compatible with Windows 7 and Windows 10 operating systems.  After downloading the software click on the setup executable and follow the instructions.

## 6.1 Main Command Window

The first screen that appears when the software is run is shown below:

Figure 6-1

## 6.2 Remote Connect via TCP/IP

Select this option if you are planning to use the Sierra Wireless RV50 modem.

### 6.2.1 Sierra Wireless RV50 Modem Configuration Settings

When configuring your modem you need to set it up to allow two way communication with the SR3017 via the RS232 serial port. This can be done using Sierra Wireless’s ALEOS software. You’ll need to set it up for Reverse Telnet/SSH.

To do this …

- Connect via the RV50 modem via an Ethernet cable to a PC.

- Log into the ALEOS software by entering http://192.168.13.31:9191 from your Web browser.

- User Name: user and Password:12345  Log In

- Select Tab <Serial> then option <Port Configuration>.

- In the Startup Mode Default field, select Reverse Telnet/SSH.

- In the Configure Serial Port field, set the speed, data, parity, and stop bits to 115200,8N1.

- Go to Services > Telnet/SSH

- In the Remote Login Server Mode field, select: Telnet

- In the Remote Login Server Telnet/SSH Port Timeout(minutes) enter: 255

- Click Apply

- Reboot the Airlink gateway

- Without the Trident Remote Communicator use a Telnet or SSH terminal client such as Putty or Teraterm to connect and verify the connection is working.

- It should be set to Autologin so use Telnet into a specified port. The default TCP port# is 12345. On logging in a Welcome screen will be displayed followed by settings ending with the text “Terminal ready”.

- If the SR3017 is connected to the RV50 and running correctly you will be able to see run status messages and communicate via the command prompt.

### 6.2.2 Telnet Screen

Once your Sierra cellular modem is configured enter OK. Each modem will have a different static IP address assigned to it. Enter that in the IP address field.

Figure 6-2

### 6.2.3 Get Directory

If you want to view the names of file you have on your SDHC card click the Get Directory button.

Figure 6-3

### 6.2.4 Offload file data and save to PC

To pull the desired files from the SDHC card and copy them to the C:\Advanced Telemetry Systems, Inc\ATS Trident Receiver\Data\” of your PC …

- Highlight the desired filenames in the upper list box

- Click on the button labeled Offload file data and save to PC.

### 6.2.5 Start Unattended Logging

To have the SR3017 automatically send data to the cellular modem click the Start Unattended Logging button.

Once your logging session has been initiated it will log into the cellular modem just before the start of each hour and retrieve the dump of the data for that previous hour. The data will be saved to “C:\Advanced Telemetry Systems, Inc\ATS Trident Receiver\Data\” in the folder containing the SR3017 unit’s ID as a “csv” file.

Figure 6-4

## 6.3 Direct Connect via Serial Port

Select this option if you are planning to connect to a Windows device via a RS232 serial port connection.

When you click on the Direct Connect via Serial Port you will be prompted for the Com port that has been assigned to your connection.

Figure 6-5

### 6.3.1 Get Directory

If you want to view the names of file you have on your SD card click the Get Directory button.

Figure 6-6

### 6.3.2 Offload file(s)

To pull the desired files from the SDHC card and copy them to the C:\Advanced Telemetry Systems, Inc\ATS Trident Receiver\Data\” of your PC …

- Highlight the desired filenames in the upper list box

- Click on the button labeled Offload file(s).

Figure 6-7

### 6.3.3 Unattended Logging

As long as this window is open unattended logging is assumed.

# 7.0 Addendum: Remote Control via Cellular Modem

Description:

This addendum describes a method for operators of the SR3017 Acoustic Receiver to remotely control the SR3017 when it is equipped with a user-supplied Sierra Wireless RV50 cellular modem.

The addendum was authored by a third party who was under contract to NOAA to design the required circuitry. It provides a wiring schematic and instructions to program an RV50 to control a normally closed relay that powers fish monitoring equipment (SR3017).

Thanks to NOAA Fisheries, Santa Cruz, CA office, for providing this documentation to ATS so it may be shared for use by ATS acoustic equipment operators.

For further information regarding this documentation, please contact ATS Sales and Service.

Remote Control via Cellular Modem

Scope:

Provide wiring schematic and instructions to program a RV50 to control a normally-closed relay that powers fish monitoring equipment (SR3017).

Procedure:

In order to control the relay, we suggest using the green (GRN) I/O line to sink current through the relay coil. To remotely control the relay, we recommend logging into the RV50’s ACEmanager interface via a public static IP address.

A schematic diagram is provided below as well as information from the manual to remotely control the relay.

The relay shown is a RIBU1C.

The diode shown is a flyback diode which suppresses the voltage spike that occurs during the removal of the sink current of the RV50. For this diode, we recommend a 1N4007 or equivalent.

It is CRITICAL that this diode be inserted correctly as it is polarity sensitive.

For ACEmanager Logon, make sure that Remote management is enabled. Then log into the unit remotely while it is online to test that you can access ACEmanager. (Change your password from 12345 to a complex one, if you haven’t done so already!)

Configure the Relay output to be OFF during initialization: (Click Gray APPLY Button)

Relay off – Normal operation (Click gray Apply button if changing. No need to reboot, even though it says so.)

Relay on – Removes Power from SR3017 (Click gray Apply button if changing. No need to reboot, even though it says so.)